Meeting Notes

4/1/2014

Sensors

- Using the UART instead of the LCD screen could cause a speed issue

- Use the logic analyzer for testing

- The problem could be the write sequences

- When reading 2 bytes, acknowledge the first byte, not the second

Xbox controller

- Have manual override for the distance sensor that will allow us to push small things out of the way.

Current Draw

- We will need about 5 A for the robot

LED’s

- Use a MOSFET to turn the LED’s on and off

- The microcontroller won’t be able to provide enough current to make them bright

- The resistor needed to limit current will probably be a power resistor

Board

- The input for the batter can be done inline

- We should be able to find a 1 A fuse that’s only about a half inch long

  - Ask Clint Manning or Dr. Sauer

  - Look into automotive fuses

- We could use a slow-blow fuse but it’s not necessary

- Automotive inline fuses make sense for the motors

- As long as you come straight out of the board, you can ignore the associated clearance issues

- When you run the DFM you get plots

  - We would have seen that there is no copper

- The plots will give you an idea of how much current the LED’s are drawing

Wheels

- Talk to the MechE’s about wheels with treads

- Will we have enough traction with 2-wheel drive?

  - Put the battery over the driving wheels and it could work

USB

- Switch to a mini instead of a micro

  - More reliable connector

Gear ratio

- Talk to professor Brownell

- Experiment with a 104:1 gear ratio