Meeting Notes

3/25/2014

Sensors

- Use pullup resistors on Picboard to get I2C.

Xbox controller

- Need to define protocol for sending messages of control.

  - Design a regular message format.

  - Open up a socket to do regular web stuff with C++.

  - TCP has the advantage of being more reliable than a shell.

  - Pi could send frames to a TCP video stream.

Board Design

- Can put a fuse on the regulator and still use the GPIO pins to power Pi. Should only be bad if we are stupid about it.

- Regulators should go in series. 12->5->3.3V

- Send list of parts through the parts order list.

- Send an email with corrections of pic schematic.

- Look into high power LED flashlight.

Batteries

- Clamp on current meter to figure out power needs.

- Measure voltage via big resistors voltage divider 300k and 100k should do.

- Use unity gain op amp buffer to keep analog signals from loading other things down.

Servos

- Should probably signal with 5V via pull up resistors.

Motors

- Currently a 26:1 gear ratio.

- Set to open drain for 5V operation with pull up resistors.