Meeting Minutes: March 30, 2016

Team: grEEn

Team Leader: David

Note Taker: Bridget

Present: Kara, Dr. Schafer

Board Design Comments:

- Need GPIO0 transmit receive - either reset or CHPD

- GPIO16 to reset pin - with CHPD

- CHPD needs to be pulled high

- 15 needs to be pulled low

- Not the right USB, use mini USB

- LED too large, need one little service MOD LED

- Get rid of switches

- Capacitors need to be surface mount not through the hole (0805/ 0603 package)

- Top of board polygons

- Ground plane on bottom - anywhere but under antenna

- Resistors shouldn’t be through hole, also need to be surface mount parts

- Heat requirement: spec calls for polygons to get rid of heat

- Position of the battery connector needs to be thought out

- Cut paper board out and try connecting different boards/ size of battery

- Pulls ups on STA/ SCI - 10k? (in spec sheet)

- Bigger resistor you have to slow down

- Use standard wire for molex connectors

- Printing in Tnames layer not copper layer

- Makes names useful ex. Con1

- Smash parts (can move labels)